REAGEN™

Dexamethasone ELISA Test Kit

RND99059

### Product Description

REAGEN™ Dexamethasone ELISA Test Kit is a competitive enzyme immunoassay for the detection of Dexamethasone, Betamethasone and Flumethasone in meat, fish, shrimp, milk, urine or blood..

The unique features of the kit are:

- High recovery (>75%), rapid, and cost-effective extraction methods

- High sensitivity (0.05 ng/g or ppb) and low detection limit (0.05 ng/g or ppb) for meat samples

- Rapid ELISA assay (less than 2 hours regardless of number of samples)

- High reproducibility

### Procedure Overview

REAGEN™ The test is based on a competitive colorimetric ELISA assay. The drug of interest has been coated in the plate wells. During analysis, sample is added along with the primary antibody specific for the target drug. If the target is present in the sample, it will compete for the antibody, thereby preventing the antibody from binding to the drug attached to the well. The secondary antibody, tagged with a peroxidase enzyme, targets the primary antibody that is complexed to the drug coated on the plate wells. The resulting color intensity, after addition of substrate, has an inverse relationship with the target concentration in the sample.

### Kit Contents, Storage and Shelf Life

REAGEN™ Dexamethasone ELISA Test Kit has the capacity for 96 determinations or testing of 42 samples in duplicate (assuming 12 wells for standards). Return any unused microwells to the foil bag and reseal them with the desiccant provided in the original package. Store the kit at 2-8C . The shelf life is 12 months when the kit is properly stored.

| Kit Contents | Amount | Storage |
| --- | --- | --- |
| Dexamethasone-Coated Plate | 1 x 96-well plate (8 wells x 12 strips) | 2-8℃ |
| Dexamethasone Standards Negative control (white CAP tube) 0.05 ng/mL (yellow CAP tube) 0.15ng/mL (orange CAP tube) 0.5ng/mL (pink CAP tube) 1.5ng/mL (purple CAP tube) 4.5 ng/mL (blue CAP tube) 10 ng/mL(spiking, optional, red cap tube) | 0.8mL 0.8mL 0.8mL 0.8mL 0.8mL 0.8mL 0.8mL | 2-8℃ |
| Dexamethasone Antibody #1 | 12mL | 2-8℃ |
| 100×HRP-Conjugated Antibody #2 | 0.25mL |  |
| Antibody #2 Diluent | 20mL |  |
| 20× Wash Solution | 28mL |  |
| Stop Buffer | 14mL |  |
| TMB Substrate | 12mL |  |
| 10X Sample Suspension Solution | 10 mL |  |
| Sample Balance Buffer I (Optional) | 10 mL |  |
| Sample Balance Buffer Ⅱ(optional) | 62g |  |
| Yogurt Sample Dilution Buffer(optional) | 10 mL |  |

If you are not planning to use the kit for over 1 month ,Dexamethasone Antibody #1 and 100× HRP-Conjugated Antibody #2 at －20℃ or in a freezer is recommended.

### Sensitivity

| Sample Type | Detection Limit (ng/g or ppb) |
| --- | --- |
| Meat((Beef/Chicken/Duck/Pork)/ Fish/Shrimp | 0.05 |
| Milk/Yogurt | 0.2 |
| Urine | 0.25 |
| Serum | 0.25 |

### Specificity

| Analytes | Cross-Reactivity (%) |
| --- | --- |
| Dexamethasone | 100 |
| Betamethasone | 100 |
| Flumethasone | 100 |
| Prednisolone | 50 |
| Prednisone | 10 |
| Methylprednisolone | <1 |
| Stanozolol | <0.1 |

### Required Materials Not Provided With the Kit

- Microtiter plate reader (450 nm)

- Incubator

- Vortex mixer, (e.g. Gneie Vortex mixer from VWR)

- 10, 20, 100 and 1000 L pipettes

- Multi-channel pipette: 50-300 L (Optional)

### Warnings and Precautions

- Do not use the kit past the expiration date.

- Do not intermix reagents from different kits or lots except for components with the same part No’s within their expiration dates. ANTIBODIES AND PLATES ARE KIT-AND LOT-SPECIFIC.

- Make sure that the 100X HRP-Conjugated Antibody #2 and Antibody #2 Diluent are mixed in correct volumes.

- Try to maintain a laboratory temperature of 20–25°C (68°–77°F). Avoid running assays under or near air vents, as this may cause excessive cooling, heating and/or evaporation. Also, do not run assays in direct sunlight, as this may cause excessive heat and evaporation. Cold bench tops should be avoided by placing several layers of paper towel or some other insulation material under the assay plates during incubation.

- Make sure you are using only distilled or deionized water since water quality is very important.

- When pipetting samples or reagents into an empty microtiter plate, place the pipette tips in the lower corner of the well, making contact with the plastic.

- Incubations of assay plates should be timed as precisely as possible. Be consistent when adding standards to the assay plate. Add your standards first and then your samples.

- Add standards to plate only in the order from low concentration to high concentration as this will minimize the risk of compromising the standard curve.

- Always refrigerate plates in sealed bags with a desiccant to maintain stability. Prevent condensation from forming on plates by allowing them equilibrate to room temperature (20 – 25℃ / 68 – 77℉) while in the packaging.

SAMPLE PREPARATION

Be sure samples are properly stored. In general, samples should be refrigerated at 2-4℃ for no more than 1-2 days. Freeze samples to a minimum of -20℃ if they need to be stored for a longer period. Frozen samples can be thawed at room temps (20 – 25℃ / 68 – 77℉) or in a refrigerator before use.

- 1 Preparation of 1X Suspension Dilution

Mix 1 volume of 10X Sample Suspension Dilution Buffer with 9 volumes of distilled water.

- Preparation of 1X Sample Balance Buffer

Add 10 mL of the Sample Balance Buffer I and 62 g of Sample Balance Buffer II (Powder) to a 250-mL plastic bottle. Add 190 mL of distilled water to the bottle, mix well to obtain a homogeneous solution.

### Meat(Beef/Chicken/Pork)/Fish/Shrimp

- Weigh out 2g of the homogenize sample and Mix well with 6ml of acetonitrile, 2mL hexane and 2.5mL 1X Sample Balance Buffer

- Vortex for 3 minutes at maximum speed.

- Centrifuge at 4,000 x g for 10 minutes at room temperature (20 – 25℃ / 68 – 77℉).

- Discard the upper hexane layer completely. Transfer 2 mL of supernatant to a new tube.Use a rotary evaporator to dry the sample in a 50 – 60℃ water bath under reduced pressure. Alternatively, the sample can be dried by blowing nitrogen gas in a 50 – 60℃ water bath..

- Add 0.5mL of 1X Sample Suspension Dilution Buffer and mix by vortexing at maximum speed for 1 min.

- Use 50 μL per well in the assay.

Note: Dilution factor: 1(Suitable for detection of 0.05 ppb to 4.5 ppb).

### Milk

Methods A:

- Take 1 mL of milk sample, add 600μL of methanol and 900μL of 1X Sample Suspension Solution.

- Vortex at maximum speed for 3 minutes.

- Centrifuge at 4,000 x g for 10 minutes or 10,000 x g for 2 minutes.

- Use 50 L of the sample in the assay.

Note: Dilution factor:2.5

Methods B:

- Weigh out 2g milk sample, Mix well,with 6ml of acetonitrile, 2mL hexane and 2.5mL 1X Sample Balance Buffer.

- Vortex for 3 minutes at maximum speed.

- Centrifuge at 4,000 x g for 10 minutes at room temperature (20 – 25℃ / 68 – 77℉).

- Discard the upper hexane layer completely. Transfer 2 mL of supernatant to a new tube.Use a rotary evaporator to dry the sample in a 50 - 60℃  water bath under reduced pressure. Alternatively, the sample can be dried by blowing nitrogen gas in a 50 – 60℃ water bath..

- Add 0.5mL of 1X Sample Suspension Dilution Buffer and mix by vortexing at maximum speed for 1 min.

- Use 50 μL per well in the assay.

Note: Dilution factor: 1(Suitable for detection of 0.05 ppb to 4.5 ppb).

### Yogurt

- Take 1 mL of yogurt sample, add 600μL of methanol, 100μL of Yogurt Balance Buffer, and 800μL of 1X Sample Suspension Solution. Vortex at maximum speed for 3 minutes.

- Centrifuge at 4,000 x g for 10 minutes or 10,000 x g for 2 minutes.

- Use 50μL of the supernatant in the assay

Note: Dilution factor:2.5

### Urine

    - Centrifuge 0.5 mL of the urine sample at 4,000 rpm for 5 minutes.

    - Take 0.2 mL of supernatant, add 0.8 mL1×sample Suspension Solution, mix well.

    - Use 50μL of the sample for the assay.

Note: Dilution factor: 5.0.

### Serum

- Blood should be collected without anticoagulant and left at room temperature for 3 hours or at 4°C overnight, to clot.

- Centrifuge at 3,000x g for 10 minutes at 4°C.

- Take 200μL of the serum from the upper layer of the blood sample, add 800μL of 1 X Sample Suspension Solution, mix well.

- Use 50μL of the sample in the assay.

Note: Dilution factor: 5.0.

DEXAMETHASONE ELISA TEST KIT PROTOCOL

### Reagent Preparation

IMPORTANT: All reagents should be brought up to room temperature before use (1 – 2 hours at 20 – 25C / 68 – 77F); Make sure you read “Warnings and Precautions” section. Solutions should be prepared just prior to ELISA test. All reagents should be mixed by gently  inverting or swirling prior to use. Prepare volumes that are needed for the number of wells being run. Do not return the reagents to the original stock tubes/bottles. Using disposable reservoirs when handling reagents can minimize the risk of contamination and is recommended.

- Preparation of 1X HRP-Conjugated Antibody #2

Mix 1 volume of the 100X HRP-Conjugated Antibody #2 with 99 volumes of Ab#2 Diluent.

- Preparation of 1X Wash Solution

Mix 1 volume of the 20X Wash Solution with 19 volumes of distilled water.

### ELISA Testing Protocol

Label the individual strips that will be used and aliquot reagents as the following example:

| Component | Volume per Reaction | 24 Reactions |
| --- | --- | --- |
| Dexamethasone Antibody #1 | 100 L | 2.4 mL |
| 1X HRP-Conjugated Antibody #2 | 150 L | 3.6 mL |
| 1X Wash Solution | 2.0mL | 48 mL |
| Stop Buffer | 100 L | 2.4 mL |
| TMB Substrate | 100 L | 2.4 mL |

    - Add 50 L of each Dexamethasone Standards and sample in duplicate into different sample wells.

    - Add 100 L of Antibody #1 and mix well by gently rocking the plate manually for 1 minute.

    - Incubate the plate for 30 minutes at room temperature (20 – 25℃ / 68 – 77℉).

    - Wash the plate 3 times with 250 L of 1X Wash Solution. After the last wash, invert the plate and gently tap the plate dry on paper towels ( Perform the next step immediately after plate washings. Do not allow the plate to air dry between working steps).

    - Add 150 L of 1X Antibody #2 solution and mix well by gently rocking the plate manually for 1 minute.

    - Incubate the plate for 30 minutes at room temperature (20 – 25℃ / 68 – 77℉).

    - Wash the plate 3 times with 250 of L 1X Wash Solution. After the last wash, invert the plate and gently tap the plate dry on paper towels ( Perform the next step immediately after plate washings. Do not allow the plate to air dry between working steps).

    - Add 100 L of TMB Substrate to each well. Time the reaction immediately after adding the substrate. Mix the solution by gently rocking the plate manually for 1 minute while incubating ( Do not put any substrate back to the original container to avoid any potential contamination. Covering the microtiter plate while incubating is recommended).

    - After incubating for 15 minutes at room temperature (20 – 25C / 68 – 77F), add 100 L of Stop Buffer to stop the enzyme reaction.

    - Read the plate as soon as possible following the addition of Stop Buffer on a plate reader with 450 nm wavelength ( Before reading, use a lint-free wipe on the bottom of the plate to ensure no moisture or fingerprints interfere with the readings).

### Dexamethasone Concentration Calculations

A standard curve can be constructed by plotting the mean relative absorbance (%) obtained from each reference standard against its concentration in ng/mL on a logarithmic curve.

Relative absorbance (%) =

Use the mean relative absorbance values for each sample to determine the corresponding concentration of the tested drug from the standard curve, in ng/mL

The following figure is a typical Dexamethasone standard curve.

REAGEN LLC

Add.: 102CommerceDr.Unit8,Moorestown,NJ08057

Tel.:856-727-0250         FAX.:856-727-0251

E-mail:reagenkits@gmail.com  reagenllc@gmail.com

Web.: www.reagen.us